AVIATION (REMOTELY PILOTED AIRCRAFT) REGULATIONS, 2018

.

ARRANGEMENT OF REGULATIONS

PART I

PRELIMINARY PROVISIONS

REGULATION

- Citation

- Interpretation

PART II

GENERAL PROVISIONS

- Applicability

- Classification of RPA

- Ownership of RPA

- Registration of RPA

- De-registration

- Airworthiness of RPA

- Insurance

- Offences and Penalties

PART III

APPROVAL AND REGISTRATION

- RPA letter of approval

- RPA safety system

- Temporary loss of RLA

- Accessory equipment

- Reporting of failures, malfunctions and defects

- Reporting of incidents or  accidents relating to RPA operation

- Authorisation for RPA operations

- Cancellation suspension or variation of authorisation

PART IV

PERSONNEL LICENSING

- General

- Requirements for the issue of an RPL

- Theoretical knowledge examination

- RPA Flight training

- Skill test

- Validity of RPL

- Pilot logbook

- Validation of foreign RPL

PART V

RPA OPERATIONS

- General obligation of an RPA owner and operator

- Prohibited or Restricted operation of RPA

- Carriage of dangerous goods

- RPA operating limitations

- Detect and avoid

- Controlled airspace

- Operation in the vicinity of aerodromes

- Operations at an aerodrome

- Releasing object or substance

- Consumption of alcohol and drugs

- Command and control

- Precautions and safety considerations

- Operations in the vicinity of people

- Operations in the vicinity of property, structures and buildings

- Radio communication requirements

- Pre-flight preparation

- Duties of the pilot

- Flight operations

- Right of way

- Use of time

- Flight folio

- Power reserves

- First aid kits

- Hand-held fire extinguishers

PART VI

PRIVATE, RECREATIONAL AND SPORTS RPA OPERATIONS

- Private RPA operations

- Training requirements for private RPA operations

- Recreational and sports RPA operations

- Validity of registered clubs

- Training Requirements for recreational and sports RPA operations

PART VII

REMOTELY PILOTED AIRCRAFT OPERATOR CERTIFICATE   (ROC)

- General requirements

- Application for an RPA Operator Certificate (ROC)

- Issuance of a ROC

- Validity and Renewal of ROC

- Amendments of an ROC

- Conducting Surveillance, tests and inspections

- Management Personnel required for RPA Commercial Operations

- Operations Manual

- Documentation and Records

- Safety Management

- Register of ROC

PART VIII

MAINTENANCE

- Continued system maintenance

- RPA maintenance

- Issuing of an RMT authorisation

- RMT logbook

PART IX

SECURITY REQUIREMENTS FOR RPA OPERATIONS

- Security Programme Requirements

PART X- TRANSITION

- Transition

GOVERNMENT NOTICE NO. …

AVIATION ACT

(CAP.70:01)

AVIATION (REMOTELY PILOTED AIRCRAFT) RPA REGULATIONS, 2019

IN EXERCISE of the powers conferred by section 19 of the Aviation Act, I, JAPPIE MHANGO, Minister of Transport and Public Works, make the following Regulations____

Citation

These regulations may be cited as the Aviation (Remotely Piloted

Aircraft) Regulations, 2019

Interpretation

2. In these Regulations unless the context otherwise requires ____

“Authority” means the Department of Civil Aviation;

“Autonomous   aircraft” means an unmanned aircraft that does not allow pilot intervention in the management of the flight

“Autonomous   operation” means an operation during which a remotely piloted aircraft is operating without pilot intervention in the management of the flight

“Command and control (C2) link” means the data link between the remotely piloted aircraft and the remote pilot station for the purposes of managing the flight.

“Conspicuity” means quality of an aircraft (e.g. lighting or paint scheme), allowing it to be easily seen or noticed by others (e.g. by pilots, ATCOs, aerodrome personnel).

“Dangerous Goods” articles means or substances which are capable of posing risk to health, safety, property or the environment and which are shown in the list of dangerous goods of the Technical Instructions or which are classifieds according to those Instructions.

“Detect and avoid” means the capability to see, sense or detect conflicting traffic or other hazards and take the appropriate action.

“Handover” means the act of passing piloting control from one remote pilot station to another.

“Operations specifications” means the authorisations, conditions and limitations associated with the RPA Operator Certificate and subject to the conditions in the operations manual.

“Remote crew member” means a crew member charged with duties essential to the operation of a remotely piloted aircraft during a flight duty period.

“Remote cruise relief pilot” means a remote flight crew member who is assigned to perform remote pilot tasks during cruise flight, to allow the remote pilot-in-command to obtain planned rest.

“Remote flight crew member” means a licensed crew member charged with duties essential to the operation of a remotely piloted aircraft during a flight duty period.

“Remote pilot” means a person charged by the operator with duties essential to the operation of a remotely piloted aircraft and who manipulates the flight controls, as appropriate, during flight time.

“Remote pilot-in-command” means the remote pilot designated by the operator as being in command and charged with the safe conduct of a flight.

“Remote pilot station” means the component of the remotely piloted aircraft containing the equipment used to pilot the remotely piloted aircraft.

“Remotely piloted aircraft (RPA)” means an unmanned aircraft which is piloted from a remote pilot station.

“Remotely piloted aircraft system (RPAS)” means a remotely piloted aircraft, its associated remote pilot station(s), the required command and control links and any other components as specified in the type design.

“Remotely piloted aircraft operating manual” means a manual, acceptable to the State of the Operator, containing normal, abnormal and emergency procedures, checklists, limitations, performance information, details of the RPA and each associated RPA model and other material relevant to the operation of the remotely piloted aircraft.

Note: The remotely piloted aircraft system operating manual is part of the operations manual.

“Required communication performance (RCP)” means a statement of the performance requirements for operational communication in support of specific Air Traffic Management (ATM) functions.

“Required communication performance type (RCP type)” means a label (e.g. RCP 240) that represents the values assigned to RCP parameters for communication transaction time, continuity, availability and integrity.

“Rest period” means a continuous and defined period of time, subsequent to and/or prior to duty, during which remote crew members are free of all duties

“RPA observer” means a trained operator and competent person designated by the operator who, by visual observation of the remotely piloted aircraft, assists the remote pilot in the safe conduct of the flight.

“RPA operator certificate (ROC)” means a certificate authorising an RPA operator to carry out specified RPA operations.

“Segregated airspace” means airspace of specified dimensions allocated for exclusive use to a specific user(s).

“Visual line-of-sight (VLoS) operation” means remote pilot or RPA observer maintains direct unaided visual contact with the remotely piloted aircraft.

ABBREVIATIONS

AIP			Aeronautical Information Publication

ATC			Air Traffic Control

ATM			Air Traffic Management

ATSU			Air Traffic Service Unit

B-VLoS		Beyond Visual Line of Sight

CTR			Control Zone

E-VLoS		Extended Visual Line of Sight

ICAO			International Civil Aviation Organization

LAR			Land and Recovery

MTOW		Maximum Take- Off Weight

RMT			Remotely Piloted Aircraft (RPA) Maintenance Technician

ROC			Remotely Piloted Aircraft (RPA) Operator’s Certificate

RLA			RPA Letter of Approval

RPA			Remotely Piloted Aircraft

RPL			Remote Pilot License

R-VLoS		Restricted Visual Line of Sight

SMS			Safety Management System

UTC			Co-Ordinated Universal Time

VLoS			Visual Line of Sight

VMC			Visual Metrological Condition

IS			Implementing Standards

Application

PART II – GENERAL PROVISIONS

3. ____ (1) This Part applies to____

- Class 1 and 2 of remotely piloted aircraft, unless otherwise approved by the Authority; and

- Persons acting as owners, operators, observers, pilots and in the performance of maintenance of RPA.

(2) For the purposes of this part, RPA may be operated for____

- Commercial operations;

- Corporate operations;

- Non-profit operations; and

- Private operations.

(3) This Part does not apply to____

- Fully Autonomous RPA, Toy and Hobby aircraft, unmanned free balloons and their operations or other types of aircraft which cannot be managed on a real-time basis during flight; and

- A model aircraft.

Categorization/

Classification

of RPA

4. ____ (1) RPA shall be classified and categorized by weight and use.

(2) There shall be three classes of RPA namely____

- Class 1: that weigh 5 kilograms and below including any payload carried by the RPA;

- Class 2: that weigh more than 5 kilograms but less than 25 kilograms including any payload carried by the RPA; and

- Class 3: that weigh 25 kilograms and above including any payload carried by the RPA.

(3) There shall be three categories of RPA____

- Category A: utilized for recreational and sports purposes only;

- Category B: utilized for private activities excluding recreational and sports purposes; and

- Category C: utilized for commercial, Corporate and Non-profit activities.

|  | Cat A | Cat B | Cat C |
| --- | --- | --- | --- |
| Classification | Recreational and Sports | Private | Commercial, Corporate and Non profit |
| Class 1 Less than 5kgs | 1A | 1B | 1C |
| Class 2 5 – 25kgs | 2A | 2B | 2C |
| Class 3 Above 25kgs | Not Authority | 3B | 3C |

Ownership of

RPA

5. (1) A person shall be eligible to own a RPA if they are____

  - A Malawian Citizen;

  - A resident in Malawi;

  - Companies, Organizations or Institutions registered in Malawi;

  - The Government of Malawi;

  - Partnerships and voluntary associations whose principal place of business is in Malawi; and

  - As the Authority may approve.

(2) Change of ownership of a RPA shall be notified to the Authority in writing within seven days.

Registration of RPA

6. ____ (1) An operator or owner of a RPA shall register it with the Authority before use.

(2) The Authority shall establish and implement a system for Registration and identification of RPA in Malawi. IS16:6

(3) Registration is not transferable

De-Registration

7. The Authority shall de-register or cancel the registration of a RPA under the following circumstances____

- Upon application of the RPA owner for purposes of registering the RPA in another State; and

- Upon destruction of the RPA or its permanent withdrawal from use.

Airworthiness of RPA

8. ____ (1) A RPA owner or operator shall ensure that all its components are in working order and in accordance with the manufacturer Manual. IS16:6(1)(c)

(2) The Authority may require RPA of a certain class and category with a type certificate to obtain a certificate of airworthiness

Insurance

9. ____ (1) A person shall not operate, or cause to be operated or commit any other person to operate RPA unless there is in force an adequate insurance policy in respect of third party risks.

(2) The minimum sum of insurance in respect of any RPA insured in accordance with sub-regulation (1) shall be notified to the Authority.

(3) An operator of RPA shall make available third party liability insurance certificate(s), in the authentic form, at the location of the RPA operator’s operational management or other location specified by the Authority.

Offences and Penalties

10. ____ (1) A pilot or the person in charge of the RPA or the owner thereof who operates, or cause to be operated or commit any other person to operate a RPA in such a manner as to endanger the safety of the air space, other aircraft, persons and property on the ground, unless he proves that the RPA was so flown without his consent, commits an offence and shall be liable upon conviction, to a fine not exceeding one million Kwacha or imprisonment not exceeding 6 months or both.

(2) A person who operates a RPA, without authorisation commits an offence and shall be liable upon conviction, to a fine not exceeding one million Kwacha or imprisonment not exceeding 6 months or both.

(3) A person who fails to display a unique identifier or the registration number commits an offence and shall be liable upon conviction, to a fine not exceeding one million Kwacha or imprisonment not exceeding 6 months or both.

(4) A person who operates, or cause to be operated or commit any other person to operate an RPA which has not been registered by the Authority commits an offence and shall upon conviction, to a fine not exceeding one million Kwacha or imprisonment not exceeding 6 months or both.

(5) Any person who fails to comply with any direction given to him by the Authority or by any authorised person under any provision of these regulations shall be deemed for the purposes of the regulations to have contravened that provision.

(6) An RPA operator who fails to comply with any of the obligations provided for by these regulations shall be liable to upon conviction, to a fine not exceeding one million Kwacha or imprisonment not exceeding 6 months or both.

(7) A person who contravenes any provision of these regulations may have his licence, certificate, approval, authorisation, exemption or other such document revoked or suspended.

PART III –APPROVAL AND REGISTRATION

Remotely Piloted Aircraft (RPA) letter of approval

11. ____ (1) No RPA shall be operated within Malawi, unless such RPA has been issued with a RPA Letter of Approval (RLA) by the Authority.

(2) An application for the issuing or renewal of an RLA shall be made to the Authority in a form and manner prescribed by the Authority. In addition, the application shall be accompanied by the appropriate fee as in the current schedule of fees.

(3) The Authority shall issue an RLA if the applicant complies with the requirements prescribed in the IS16:11.

(4) An RLA shall be valid for a period of 12 months.

Remotely  Piloted Aircraft (RPA) Safe System

12. ____ (1) An applicant for the issue of an RLA, shall provide the Authority with-

- Documentation regarding the standard to which the RPA was designed;

- Equivalent documentation that demonstrates a level of safety acceptable to the Authority; and

- Documentation demonstrating system safety as prescribed in the requirements as prescribed in the IS16:11(3)

(2) Altimeter ____

- Except as provided in the IS16:11(3), an RPA shall be equipped with an altimetry system or equivalent that is capable of displaying to the operator on the RPA, the altitude and height of the RPA above ground level; and

- An RPA that is not equipped with an altimetry system or equivalent shall be operated under R-VLoS only.

Temporary Loss of RLA

13. ____ (1) Any failure to maintain a RPA in an airworthy condition as defined by the appropriate requirements mandated by the Authority and manufacturer of the RPA shall render the RPA ineligible for operation until restored to an airworthy condition.

(2) When a RPA has sustained damage, the Authority shall judge whether the damage is of a nature such that the aircraft is no longer airworthy as defined by the appropriate design, specifications and manufacturer’s documents.

Accessory Equipment

14. All accessory equipment mounted on the RPA shall be approved by the Authority after evaluation of a technical request.

Reporting Failures Malfunctions and Defects

15. Owners or operators of RPA over 20kg MTOW shall as soon as practical, but in any case within 3 days, report to the Authority any failures, malfunctions, or defects in flight that result in at least the following____

    - Fire to the propulsion system, any shut down, flameout or seizure, where applicable, internal or external to the propulsion system, over speed, propeller feathering, FOD ingestion, fuel leakage, premature change of propulsion units and other condition prescribed by the Authority;

    - RPA component that causes accumulation or circulation of smoke, vapour, or toxic or non-toxic fumes in any compartment;

    - Where applicable, brake system components failure that result in loss of brake actuating force when the RPA is in motion on the ground;

    - RPA structure that requires major repair; Cracks, permanent deformation, or corrosion of aircraft structure, if more than the maximum acceptable to the manufacturer or the Authority;

    - RPA components or systems malfunctions that result in taking emergency actions during flight (except action to shut down an engine);

    - Each interruption to a flight caused by known or suspected technical difficulties or malfunctions; and

    - Abnormal vibration or buffeting caused by a structural or system malfunction, defects, or failure; and a failure or malfunction of more than one attitude, airspeed, or altitude instrument during a given operation of the aircraft.

Reporting of Incidents and Accidents relating to RPA operation

16. ____ (1) RPA operator shall report all serious incidents and accidents to the Authority within 3 days in the form and manner as prescribed by the Authority as prescribed in the IS16:16.

(2) The Authority shall be responsible for investigation of all RPA accidents and serious incidents.

Authorisation for RPA operations

17. ____ (1) A person shall not conduct an RPA flight commencing at a place within Malawi and terminating at a place outside without authorisation from the State of destination or any other State over whose airspace the RPA shall fly.

(2) A person shall not conduct a RPA flight commencing at a place outside Malawi and terminating at a place within Malawi or overflying the Malawian airspace without authorisation from the Authority

(3) Unless otherwise specified by the Authority, the request for authorization shall include the items prescribed in IS16:17

Cancellation suspension or variation of authorisation

18. Authority may, in the interest of safety and security, cancel, suspend or vary any authorisation granted under these regulations

PART IV –PERSONNEL LICENSING

General

19. ____  (1) No person shall act as a remote pilot, except when undergoing a skills test or receiving flight instruction, unless the person is in possession of a valid Remote Pilot Licence (RPL) issued by the Authority.

(2) A remote pilot licence may be issued for the following categories____

      - Aeroplane;

      - Helicopter;

      - Multirotor;

      - Hybrid; and

      - Airship.

(3) Individual pilots shall have the following ratings endorsements on their personal licence____

(a) VLoS:	visual line-of-sight operations;

(b) E-VLoS:	extended visual line-of-sight operations; and

(c) B-VLoS:	beyond visual line-of-sight operations.

Requirements for the issue of an RPL

20. ____ (1) An applicant for an RPL shall____

- Not be less than 18 years of age;

- Undergo medical assessment at an approved aviation medical examiner;

- Hold at least Proficiency in Radiotelephony (Aeronautical);

- Provide proof of the ability to speak the English language at proficiency level 4 or higher; and

- Have completed a course of training approved by the Authority and passed a knowledge and skill test.

(2) Where applicable RPA pilot shall hold an appropriate rating for the type of operations they will perform including____

- Type Rating;

- Instrument Rating;

- Night Rating; and

- Instructor Rating.

(3) An application for an RPL must be made to the Authority on the appropriate form within 30 days of completing the practical skill test.

(4) A person undergoing training to qualify for a remote pilot license or rating shall not act as solo remote pilot of an RPA unless under the supervision of, or with the authority of, an authorised RPA instructor.

(5) RPL requirements are prescribed in IS16:20

Theoretical knowledge examination

21. ____ (1) The theoretical knowledge examination applicable to the category of licence sought must be passed within 90 days preceding the skill test.

(2) The theoretical knowledge examination shall be conducted at a test centre accredited by the Authority, except as provided for in the requirements as prescribed in the Malawi Aviation Regulations.

(3) The syllabi as well as other requirements for the examination shall be as prescribed in the IS16:20(1) to (6).

(4) Approval shall be obtained from the Authority before any foreign theoretical training or theoretical knowledge examination is undertaken if such training or knowledge is to be accredited towards a Remote Pilot Licence (RPL).

RPA Flight training

22. ____ (1) The flight training syllabi for the different categories of licence shall be as prescribed in the IS16:20(7) to (11).

(2) The organisation conducting the training shall issue a certificate stating that flight training has been successfully completed.

(3) All flight training shall be conducted with an RPA of the same category for which the licence is sought.

(4) Approval shall be obtained from the Authority before any foreign flight training is undertaken if such training is to be accredited towards a remote pilot licence (RPL).

Skill test

23. ____ (1) The skill test for an RPL shall be conducted within 60 days of completing the flight training by an examiner accredited by the Authority.

(2) The skill test shall be conducted with an RPA of the same category for which the licence is sought.

(3) The holder of the remote pilot licence (RPL) shall submit the skill test form to the Authority within 30 days of the skill test.

(4) The skill test shall include the applicable sections for the extended visual line of sight (E-VLoS) and beyond visual line of sight (B-VLoS) ratings if one or more of these ratings are sought.

Validity of Pipl

24. ____ (1) An RPL shall be valid for 24 months unless suspended, revoked or surrendered.

(2) RPL shall be valid subject to validity of the holders medical certificate and proficiency check

(3) The holder of a remote pilot licence (RPL) shall renew within 30 days before expiry.

Pilot logbook

25. ____ (1) The holder of remote pilot licence (RPL) shall maintain a record of flight time, instrument time, simulation time and instruction time.

(2) The pilot shall retain all pilot logbooks for at least 60 months from the date that person no longer holds a valid pilot licence.

(3) The holder of an RPL must make the logbook available for inspection upon a request by the Authority

Validation of Foreign RPL

26. For Foreign RPA brought into Malawi for a limited period, a validation of an equivalent certificate to the RLA shall not extend beyond the validity of the foreign equivalent document but not exceeding 12 months

PART V –RPA OPERATIONS

General obligation of an RPA owner and operator

27. ____ (1) The RPA operator shall be responsible for the safe conduct of its operations.

(2) The RPA operator shall comply with all requirements established by the Authority regarding its operation.

(3) The RPA operator shall be responsible for contracted services from providers as necessary, to carry out its operations.

(4) Responsibility for operational control shall rest with the registered owner or operator of the RPA.

(5) The RPA owner or operator shall ensure that it is registered in accordance with the provisions of these regulations

Prohibited or Restricted operator of RPA

28. ____ (1) A person shall not operate an RPA____

- In manner that endangers other aircraft, persons or property;

- In prohibited areas.

- In restricted, danger areas or any other area notified by the Authority except with the permission of and in accordance with any conditions imposed by the Authority.

- No person shall use a public road as a place of landing or take-off of an RPA, except____

- Unless approved by the Authority; and

- When approved by the relevant authority.

Carriage of Dangerous goods

29. ____ (1) No person may use an RPA to carry dangerous goods as cargo, except by the holder of an ROC and as approved by the Authority in the ROC holder’s Operations Specifications.

(2) In accordance with the technical instructions for the safe transport of dangerous goods by air issued by the Council of ICAO with any variations with those instructions that the Authority may from time to time mandate and provide notification of dangerous goods to ICAO

(3) A person shall not operate an RPA above 400 feet Above Ground Level and within 30 meters of any person, vessel, vehicle or structure which is not under the control of the person in charge of the RPA.

RPA Operator Limitation

30. ____ (1) Notwithstanding the provisions of sub regulation (1), operations for private and commercial categories of RPA may be conducted at such heights and lateral distances as the Authority may approve.

(2) A person shall not operate an RPA unless approved by the Authority____

- In conditions other than Visual Meteorological Conditions (VMC);

- At night; and

- Beyond Visual Line of Sight (B-VLoS).

Detect and avoid

31. Notwithstanding the provisions of regulation 30(3) (c) RPA to be operated Beyond Visual Line of Sight shall be equipped with a detect and avoid system unless approved by the Authority.

Controlled airspace

32. ____ (1) No RPA may be operated in controlled airspace, except by the holder of an ROC and as approved by the Authority

(2) The Authority may approve an RPA operation in controlled airspace as contemplated in Aviation (Rules of the Air and Air Traffic Control) Regulations currently in force, only in____

- VMC in an ATZ and CTR below 400feet; and

- Subject to compliance with the conditions prescribed in the Malawi Aviation Regulations currently in force.

(3) RPA operating in controlled airspace shall have the same ability to automatically detect-and-avoid traffic/ obstacles, as other aircraft while moving through the air.

- ATC communication____

- RPA pilots shall ensure that ATC is made aware of any operations that shall take place in areas which are likely to affect manned and controlled air traffic;

- The ATC shall establish procedures, acceptable to the Authority, for integration of RPA operation into the airspace to ensure aviation safety. Such procedures shall include communication and surveillance detection; and

- Procedures referred to in sub-regulation (b) shall prescribe required information to be passed to ATC by RPA pilot before and during RPA operations.

Operation in the vicinity of Aerodrome

33. A person shall not operate a RPA____

- within 10 kilometers of an aerodrome from the aerodrome reference point for code C, D ,E and F aerodromes;

- within 7 kilometers of an aerodrome from the aerodrome reference point for code A and B aerodromes;

- on approach and take-off paths;

- within the vicinity of navigation aids;

- within the aerodrome traffic zone;

- within terminal traffic holding patterns;

Except with approval from the Authority.

Operations of an Aerodrome

34. The Authority may upon approval of RPA operation at an Aerodrome____

(a) Impose operating restrictions on the approval in the interest of safety;

(b) Publish details of the approval in the appropriate element of the Aeronautical Information Publication (AIP); and

(c) Revoke or change the conditions that apply to such approval and publish details of any revocation or change in conditions in the appropriate element of the AIP.

Releasing object or substance

35. No object or substance shall be released, dispensed, and dropped, delivered or deployed from an RPA unless approved by the Authority.

Consumption of aclcol and drugs

36. No remote pilot, observer or RMT shall____

- Consume alcohol less than 8 hours prior to reporting for duty;

- Commence a duty period while the concentration of alcohol in any specimen of blood taken from any part of his or her body is more than 0.02 grams per 100 millilitres;

- Consume alcohol or any psychoactive substance during the duty period or whilst on standby for duty; and

- Commence duty period while under the influence of alcohol or any psychoactive substance having a narcotic effect.

disaster

Command and Control

37. A RPA owner or operator shall ensure that he or she has command and control at all times during the flight.

Precautions and safety considerations

38. ____ (1) No person shall operate an RPA unless____

- The RPA is in a fit-to-fly condition;

- The pilot is the holder of a licence issued in terms of this Part of regulation; and

- The RPA station is compatible and interoperable with the RPA it is connected to in all phases of flight.

(2) No person shall operate an RPA in a negligent or reckless manner so as to endanger the safety of any person, property or other RPA/aircraft in the air or on the ground.

(3) The operator shall, in the best interest of safety, ensure that certain RPA operations are supplemented with additional personnel for non-flying duties, such that the remote pilot can maintain control and situational awareness in respect to positioning and collision avoidance

Operations in the vicinity of people

39. No person shall operate an RPA directly overhead any person or group of people or within a lateral distance of 30 m from any person, unless____

- The operator is the holder of an ROC and the operation has been approved by the Authority in their operations manual;

- Such person is the operator of the RPA or such person is under the direction of the operator of the RPA; or

- Such person or group of people forms part of the operations of the RPA, and is under control of the operator of the RPA, and adequate provisions have been made for their safety.

Operations in the vicinity of property structure and building

40. No RPA shall be operated within a lateral distance of 30m from any structure or building, unless____

    - The operator is a holder of an ROC and the operation has been approved by the Authority.; and

    - Permission is obtained from the owner of such structure or building.

(2) An operator conducting an operation as contemplated in subsection (1) above, shall take such measures as are necessary to ensure the safety of all persons on the ground accessing such building or in the vicinity of such structure.

Radio Communication requirements

41. ____ (1) If operations are to be conducted within the 3 nautical miles boundary of an aerodrome, except for R-VLoS operations, no RPA shall be operated unless the pilot has a functioning air-band radio in his possession, tuned to the frequency or frequencies applicable to the Air Traffic Service Unit (ATSU) providing services or controlling such area or airspace or to aircraft in such area or airspace.

(2) The air-band radio shall have the required output and be configured in such a way that the range, strength of transmission and quality of communication extends beyond the furthest likely position of the RPA from the pilot.

(3) For VLoS, E-VLoS and B-VLoS operations, the pilot shall, using the registration of the RPA as a call-sign, make the required radio calls, indicating the altitude, location and intended operation of the RPA in that area and at such intervals as are required in order to ensure adequate separation from other aircraft is maintained.

(4) For approved RPA operations in controlled airspace, the pilot shall maintain radio contact, using the registration of the RPA as a call-sign, with the relevant ATSU, and acknowledge and execute such instructions as the ATSU may give at any time during the operation of the RPA.

(5) The Authority is responsible for the allocation of the operating frequency to users in the Microwave band

- All RPA operators shall apply to the Authority for Microwave band frequency allocation.

Pre-flight Preparation

42. A pilot shall complete the pre-flight preparations prior to each flight, as prescribed in the IS16:63 Part B(2)(2.4)

Duties of the pilot

43. ____ (1) The pilot is accountable for safe operation of the RPA.

(2) The pilot of an RPA shall, on each flight, operate such aircraft in accordance with the manual.

(3) The pilot of an RPA is responsible for separation and avoidance of the RPA from other aircraft and any other obstacles and hazards.

(4) The pilot of an RPA shall pilot such RPA in a manner so as to minimize hazards to persons and property on the ground, and other aircraft in the air.

(5) The pilot shall ensure that at least one observer is used for E-VLoS operations.

Flight operations

44. ____ (1) The RPA shall be operated in such a way that safe separation from other aircraft is maintained and that adequate obstacle clearance is ensured, during all phases of the flight.

(2) The pilot of an RPA shall ensure that the Launch and Recovery (LAR) area is safe and of the appropriate dimensions, free from obstacles and has adequate surface conditions, with regard to the type of operation, the size of the RPA, the RPA performance and external factors.

An RPA shall give way to manned aircraft.

Right of way

45. ____ (1) The RPA shall avoid passing over, under or in front of manned aircraft, unless it passes well clear and takes into account the effect of aircraft wake turbulence.

(2) When two RPAs are approaching head-on or approximately in a way that there is danger of collision, each RPA shall alter its heading to the right.

(3) When two RPAs are converging at approximately the same level, the RPA which has the other RPA on its right, shall give way.

(4) An RPA which is being overtaken has the right of way, and the one overtaking shall alter its heading to keep well clear.

(5) RPA is to descend by a minimum of 100 feet as soon as a manned aircraft is detected in the immediate vicinity to ensure separation.

Use of time

46. ____ (1) For the purposes of reporting and recording time, Co-ordinated Universal Time (UTC) shall be used and shall be expressed in hours and minutes and, when required, seconds of the 24-hour day beginning at midnight.

(2) A pilot shall have a time piece synchronised with UTC prior to operating an RPA in controlled airspace and at such other times during the flight as may be necessary.

(3) Wherever time is utilised in the application of data link communications, it shall be accurate to within 1 second of UTC.

Flight Falio

47. ____ (1) The owner or operator of an RPA shall ensure that the RPA has a flight folio or any other similar document which meets the requirements of and contains the information as prescribed in the Aviation Regulations currently in force, and the flight folio shall be accessible at the remote pilot station all times during flight.

(2) The flight folio shall be kept up-to-date and maintained in a legible manner by the remote pilot.

(3) All entries shall be made immediately upon completion of the occurrence to which they refer.

(4) In the case of maintenance being undertaken on the RPA, the entry shall be certified by the person responsible for the maintenance.

(5) Without detracting from the generality of the Aviation Regulations currently in force, the remote pilot shall____

        - Maintain fuel or charging records to enable the owner or operator to ascertain that, for each flight under his or her control, the requirements of regulation are complied with;

        - Enter the fuel, charging and oil records referred to in the Aviation Regulations currently in force in the flight folio; and

        - Maintain oil records to enable the owner or operator to ascertain that trends for oil consumption are such that an RPA has sufficient oil to complete each flight.

(6) The Flight Folio shall incorporate the following information for each flight in the form of a flight folio as outlined in IS16:47

Power reserves

48. ____ (1) During VLoS operations, the remote pilot shall ensure that the RPA has enough fuel or electrical charge to complete the flight, plus a reserve of at least 10%.

(2) During B-VLoS operations, the remote pilot shall ensure that the RPA has enough fuel or electrical charge to complete the intended flight plus a reserve of at least 10%.

First Aid kits

49. All operators of an RPA shall have a first aid kit available within the remote pilot station

Hand-held Fire Extinguishers

50. All operators of an RPA shall have a hand held fire extinguisher within the remote pilot station.

PART VI –PRIVATE RPA OPERATIONS

Private RPA Operations

51. A person shall not operate RPA for private purposes except with prior authorisation issued by the Authority and subject to the conditions contained therein

Training requirements for private RPA operator

52. The RPA pilots will be trained in accordance with training requirements in IS16:20.

Recreational and sports RPA Operations

53. ____ (1) RPA operations for recreation and sports purposes shall be conducted within registered clubs or associations which are approved by the Authority as set out in the IS16:53.

(2) The Authority shall develop a system for approval of clubs including requirements for composition, documentation and club rules and regulations

(3) The clubs referred to in sub-regulation (1) shall provide the Authority with details of their operation areas and times for approval

(4) The Authority shall segregate and notify through the applicable element of the AIP of such airspaces designated for use by RPA operators, including limitations that may apply

Validity of registered clubs

54. Validity of the registered clubs shall be 12 months.

Training Requirements for recreational and sports RPA operations

55. ____ (1) Clubs referred to in regulation 53 herein shall prescribe minimum training requirements for RPA operation under the club.

(2) Training requirements referred to in sub-regulation (1) above shall be documented and submitted to the Authority for acceptance.

PART VII- REMOTELY PILOTED AIRCRAFT

General requirements

OPERATOR CERTIFICATE (ROC)

56. ____ (1) No person shall engage in commercial, corporate and non-profit operations unless the operator holds a valid RPA Operator Certificate (ROC) issued by the Authority.

(2) The ROC shall authorise the operator to conduct RPA operations in accordance with the conditions and limitations detailed in the operations specifications attached to the ROC.

(3) The issuance of an ROC by the Authority is dependent upon the RPA operator demonstrating an adequate organisation, method of control and supervision of flight operations, training programme as well as ground handling and maintenance arrangements consistent with the nature and extent of the operations specified and commensurate with the size, structure and complexity of the organisation.

(4) A commercial RPA operator shall establish and implement a safety management system (SMS) in accordance with their operational requirements.

Application for an RPA Operator Certificate

57. ____ (1) A prospective operator applying to the Authority for an RPA Operator Certificate (ROC) shall submit an application in a form and manner prescribed by the Authority.

(2) A prospective operator shall make the application for an initial issue or reissue of an ROC at least 60 days before the date of the intended operation.

(3) No RPA shall be registered under more than one ROC.

Issuance of a ROC

58. The Authority may issue an ROC to the prospective operator if that applicant meet the following requirements____

(59) Has its principal place of business and it is registered in Malawi.

(60) Meets the requirements of the applicable regulations and for the holder of an ROC.

(61) Is properly qualified and adequately staffed and equipped to conduct safe operations in commercial operations of the RPA.

(62) Hold a security clearance issued by the relevant authority.

(63) Has an operations manual as prescribed in IS16:63.

Validity and Renewal of ROC

59. ____ (60) An ROC shall be valid for 12 months from the date of issue or renewal unless____

(a) The holder surrender the ROC to the Authority

(b) The Authority amends, suspends, revokes or otherwise terminates the certificate.

(61) An ROC which is suspended or revoked shall be returned to the Authority within 7 days from the date on which the ROC is suspended or revoked.

(62) The holder of an ROC shall, at least 30 days immediately preceding the date on which such certificate expires, apply for the renewal of such certificate.

(63) A prospective operator for an ROC which has expired shall make an initial application

Amendments of an ROC

60. ____ (1) The Authority may amend an ROC if the____

- Authority determines that the amendment is necessary for the safety of commercial RPA operations.

- ROC holder applies for an amendment and the authority determines that the amendment is necessary.

(2) The ROC holder shall operate in accordance with the amendment unless it is subsequently withdrawn.

Conducting Surveillance tests and inspections

61. The ROC holder shall permit the Authority to conduct surveillance, tests, and inspections on the ROC holder to ensure continued eligibility to hold an ROC and associated approvals.

Management Personnel required for RPA commercial Operations

62. ____ (1) RPA operator shall have an Accountable Manager acceptable to the Authority, with corporate authority for ensuring that all necessary resources are available to support ROC operations.

(2) The ROC holder shall have sufficient qualified and competent personnel for the planned tasks and activities to be performed in accordance with the applicable requirements.

(3) An ROC holder should establish initial and recurrent training to ensure continuing competence of its personnel.

Operations Manual

63. An ROC holder shall develop and submit to the Authority for approval an Operations Manual as set out in the IS16:63.

Documentation and Records

64. ____ (1) An ROC holder shall establish a system of record-keeping that allows adequate storage and reliable traceability of all activities developed, covering in particular____

- Lines of responsibility and accountability

- Safety policy

- Identification of aviation safety hazards encountered by the activities of the operator, assessment and mitigation of the associated risks, including taking actions and verifying their effectiveness.

- Personnel training and competence

- Quality, safety and security management records

(2) The format of the records shall be specified in the ROC holder’s operations manual.

(3) Records shall be stored for at least 2 years in a manner that ensures protection from damages, alteration and theft.

Safety Management

65. ____ (1) The ROC holder shall establish a safety management system commensurate with the size of the organisation, structure and the complexity of its operations. IS16:63 Part A(2)

(2) The safety management system shall include____

- A process to identify actual and potential safety hazards and assess the associated risks;

- A process to develop and implement remedial action necessary to maintain an acceptable level of safety;

- Provision for continuous and regular assessment of the appropriateness and effectiveness of safety management activities.

Register of ROC

66. ____ (1) The Authority shall maintain a register of all ROC certificate.

(2) The ROC shall not be transferable.

PART VIII – MAINTENANCE

Continued system maintenance

67. ____ (1) An operator of RPA shall be compliant with the manufacturer’s instructions for continued equipment maintenance through actions or inspections

(2) The operator shall submit to the Authority for approval, a maintenance programme for RPA

(3) The maintenance and inspection requirements are prescribed in IS16: 67

RPA maintenance

68. The maintenance on an RPA or any component thereof shall be carried out by the following persons____

- In respect of an RPA classified as a Class 3 and higher, the holder of a valid RMT authorization; or

Issuing of an RMT authorisation

In respect of an RPA classified as Class 2 and lower, the ROC holder: provided that the holder can demonstrate to the satisfaction of the Authority, its ability to perform the required maintenance on the RPA.

69. ____ (1) An applicant for the issuing or renewal of an RMT authorisation shall____

- Be not less than 18 years of age;

- Be a Malawian citizen or in possession of a valid permanent residence permit or valid temporary work permit with a letter of employment;

- Shall have successfully completed appropriate training, provided by____

- An organisation approved by the competent authority in the country where the training organisation is located;

- Training provided by an approved original equipment manufacturer; or

- A training facility approved by the Authority.

- Demonstrate to the Authority, the ability to perform maintenance functions where no training for the particular RPA is offered or available.

(2) An application for the issuing of an RMT authorisation shall be made in a form and manner as prescribed by the Authority and accompanied by the appropriate fee as prescribed in the current schedule of fees.

(3) The Authority shall issue an RMT authorisation if the applicant complies with the requirements.

(4) The holder of an RMT authorization shall not exercise privileges other than the specific privileges for which the authorization is issued.

(5) An RMT authorisation shall be valid for a period of 24 months

Any person responsible for the maintenance of RPA shall maintain a personnel logbook and shall record therein all work carried out in an RPA and its components.

RMT logbook

70. ____ (1) The form of and information to be contained in a logbook, and the manner in which such logbook shall be maintained, is as per IS16:70

(2) No alterations of a logbook shall be made once it is signed off by a designated person

PART IX- SECURITY REQUIREMENTS FOR RPA OPERATIONS

Security Programme Requirements

71. ____ (1) A person or a club shall not operate a RPA without Operator Security Procedures unless accepted by the Authority.

(2) An RPA operator shall specify the security measures, procedures and practices to be followed by the operator to protect pilots and facilities from acts of unlawful interference.

(3) An RPA operator shall carry out and maintain security measures including identification and resolution of suspicious activity that may pose a threat to civil aviation____

- at a remote pilot station;

- on an RPA; and

- at any facility under the control of the RPA operations.

(4) The specific security measures referred to in sub-regulation (3) shall provide____

(a) That the premises used for preparing, storing, parking including RPA ground station shall be secured at all times against unauthorized access;

(b) For protection of critical information technology and communication systems used for operations purposes from interference that may jeopardize the security of civil aviation;

(c) For protection of flight documents;

(d) That Commercial Operators requesting to operate with a camera shall be required to include details of the camera usage in the application for Security review and approval;

(e) Requirements for checks and searches of specific areas and accessible compartments of the interior and exterior of RPA; and

(f)  That persons engaged in RPA operations are subject to recurrent background checks and selection procedures and are adequately trained.

PART X- TRANSITION

Saving and Transitional Provisions

72. Any RPA operator prior to the publication of these regulations shall be registered with the Authority within twelve months of such publication.

Made this		day of				, 2019.

J. Mhango

Minister of Transport and Public Works

(File No. …………….)

IMPLEMENTING STANDARDS OF REMOTELY PILOTED AIRCRAFT (RPA)

IS16: 6: REGISTRATION OF RPA

- APPLICATION FOR REGISTRATION OF A RPA

  - Any person who wishes to register a RPA in Malawi shall submit to the Authority an application for the registration of the RPA in a form and manner prescribed by the Authority.

  - Each applicant shall____

- Certify as to compliance with registration eligibility;

- Show evidence identifying ownership, and submit applicable documentation as shall be required by the Authority;

- Complete and sign the form in ink; and

- Submit to the Authority the completed application form including a fee as shall be stated in the Schedule of Fees published by the Authority.

  - Upon an applicant meeting all requirements for registration, a certificate of registration or other document may be issued by the Authority.

  - Completed application forms for registration of a RPA in Malawi shall include or be accompanied by such particulars and evidence relating to the RPA and ownership or hire or lease or partnership (local or foreign)

  - In particular, the application shall include the proper description of the RPA according to the “General Classification of a RPA” set forth in these regulations.

  - Upon receiving an application for registration of a RPA in Malawi and being satisfied that the RPA may properly be so registered, the Authority may register the RPA, where ever it may be, subject to a physical examination and other inspections by the Authority

- ISSUANCE OF CERTIFICATE OF REGISTRATION FOR A RPA

  - After processing and successfully evaluating the applicant’s completed form for registration of RPA, the Authority may issue a Certificate of Registration of a RPA or, where applicable, an equivalent document as shall be determined by the Authority

  - The certificate of registration for a RPA, in wording and arrangement, shall be in a format as in ICAO Annex 7

  - Where some other document is issued by the Authority equivalent to a certificate of registration of a RPA, the form and manner shall be determined by the Authority from time to time.

  - The Certificates of Registration, or equivalent document, issued by the Authority shall be in English.

- RESPONSIBILITY OF A REGISTERED OWNER OF A RPA

  - Any person, who is an owner or an operator of a RPA registered in Malawi, shall be responsible for compliance with these regulations.

  - The registered owner or operator shall forthwith inform the Authority in the writing of____

- Any change in the particulars which were furnished to the Authority upon application being made for the registration of the RPA;

- The destruction of, or serious damage to, the aircraft, or its permanent withdrawal from use;

- In the case of a RPA registered under Hire Purchase Arrangement, the termination of the hire-purchase agreement; and

- When the registered owner of the RPA ceases to be the owner of that RPA.

  - Any person who purchases or acquires a RPA registered in Malawi shall within 28 days inform the Authority in writing to that effect before further flight of the RPA.

  - If a certificate of registration of a RPA, or other equivalent document issued by the Authority, is lost, stolen or destroyed or so damaged that the particulars are no longer clearly legible, the holder shall notify the Authority within 14 days of becoming aware of the loss, theft, destruction or damage and the circumstance thereof, whereupon the Authority may, if it is satisfied that the certificate is lost, stolen, destroyed or damaged, issue a copy of the certificate to the holder on payment of an appropriate fee.

  - Any person or Company or Organization or Institution  that is a registered owner of a RPA shall be responsible for____

- All costs associated with scheduled and unscheduled maintenance of said RPA;

- Any maintenance prescribed by the Authority following a determination on basis of safety, regulatory requirements and other continuing airworthiness aspects;

- Meeting all costs associated with an incident or accident investigation and other aspects associated with such an activity as the Authority shall specify; and

- All other aspects deemed by the Authority to ensure that the owned RPA complies with these regulations.

  - Insurance of a RPA

The owner or operator of a RPA registered, or where a foreign owner of a RPA partners a local person, company, organization or institution, shall hold an insurance, which shall be subject to scrutiny by the Authority, to cover the issues raised in this part and other related aspects such as clearance from an active runway or taxiway or other airfield section following an incident or accident when directed by the Authority.

- CLASSIFICATION OF RPA FOR PURPOSES OF REGISTRATION

RPA CLASIFICATION TABLE

| Class | line-of-sight | Kinetic energy* (kJ) | Height (AGL)(ft) | MTOW (kg) | Document issued by DCA after registration |
| --- | --- | --- | --- | --- | --- |
| Class 1 | R-VLoS/VLoS /E-VLoS/ | A | h < 400 | m < 5 | Approval Letter |
| Class 2 | R-VLoS/VLoS /E-VLoS/ | Ek < 34 | h < 400 | m < 20 | Certificate of Registration |
| Class 3 | VLoS | Ek > 34 | h < 400 | m < 20 | Certificate of Registration |
| Class 4 | RLoS | Ek > 34 | h < 400 | m < 150 | Certificate of Registration |
| Class 5 | VLoS | Any | h > 400 | m < 150 | Certificate of Registration |
| Class 6 | RLoS | Any | h > 400 | m < 150 | Certificate of Registration |
| Class 7 | Any | Any | Any | m > 150 | Certificate of Registration |
| Class 8 | Reserved | Reserved | Reserved | Reserved |  |

H - means height above the surface

Ek= 1/2∗Mass ∗ (1.4vmax)2

- NATIONALITY AND REGISTRATION MARKS

  - General

- No person may operate a RPA registered in Malawi unless it displays nationality and registration marks in accordance with the requirements of this section;

- The letter or letters used to identify the RPA nationality as of Malawi shall conform to the requirements outlined in Annex 7 to the Convention on International Civil Aviation. This is to be followed by a series letters assigned by the Authority; and

- Unless otherwise authorized by the Authority, no person may place on any RPA a design, mark, or symbol that modifies or confuses the nationality and registration marks. Marks shall not be used which might be confused with the International Five Letter Code of Signals, Part II, the three-letter combinations beginning with Q used in the Q Code or Distress Codes or other similar, urgent codes.

  - Markings

Permanent marking of a RPA nationality and registration shall____

- Be painted on a RPA or affixed by other means ensuring a similar degree of permanence;

- Have no ornamentation;

- Contrast in color with the background;

- Be legible; and

- Be kept clean and visible at all times.

  - Display of Marks: General

Due to different sizes and shapes of RPAs, the Authority will issue the specifics on display, position and size of registration marks upon physical inspection of the applicant’s RPA.

    - General

- Generally, each owner shall display on the RPA marks consisting of the Roman capital letter(s) [denoting nationality of Malawi, followed by the registration mark of the RPA in, Roman capital letters.

- A hyphen shall be placed between the national mark and the registration mark.

- If because of the RPA configuration, it is not possible to mark the RPA in accordance with this Sub-Part, the owner shall apply to the Authority for a different location of marks on the RPA.

  - Size of Marks

The precise size of marks shall be given after a physical inspection by an inspector from the Authority.

  - Location of Marks

The precise location of marks shall be given after a physical inspection by an inspector from the Authority.

  - Special Cases for size and location of marks

In special cases for size and location of marks, and where the Authority is not issuing a Certificate of Registration, the Authority will give precise guidance on size and location on a case by case basis.

- REGISTER OF RPA

  - The Authority shall keep and maintain a Register of RPA in Malawi

  - The Register of RPAs shall include in the register the following____

- The serial number of the certificate or other document issued;

- The nationality mark of the RPA and the registration mark assigned to it by the Authority where applicable;

- The name of the constructor of the RPA and its designation;

- Maximum takeoff weight;

- The serial number and year of manufacture of the RPA;

  - The register of RPA shall hold other particulars deemed applicable by the Authority among which shall be____

- The name and address of every person who has a legal interest or other interest or share therein; and

- In case of a RPA which is subject of a Hire Purchase Agreement or other partnership (temporary or a foreign organization in partnership with a local, the particulars thereof.

- GENERAL

  - Sale of RPA

    - On sale or resale of an RPA, the seller must display and provide a notice, to notify the purchaser of the following information:

“Note: The operation of Remotely Piloted Aircraft in Malawi is regulated under the Aviation (Remotely Piloted Aircraft) Regulations, as amended. Operations as hobbyist are subject to terms of relevant and approved “association/clubs” constitution or non-type certificated aircraft requirements.

Private use is restricted in terms of the following____

- RPA shall only be used for an individual’s personal and private purposes where there is no commercial outcome, interest or gain;

- The RPA shall only be operated over property for which the operator has ownership or permission;

- The RPA shall only be used in Restricted Visual Line of Sight which means within 500m of the pilot, and never to exceed the height of the highest obstacle within 300m of pilot, during which the pilot can maintain direct unaided visual contact with the device to manage its flight and collision avoidance; and

- The pilot shall observe all statutory requirements relating to liability, privacy and any other laws enforceable by any other authorities.

For all other use____

(i) The RPA must first be approved by the Authority for use by way of an RPA Letter of Approval(RLA);

(ii) All RPA must be registered by the Authority prior to use; or

(iii) An RPA may only be operated in terms of Aviation (Remotely Piloted Aircraft) Regulations which includes specific requirements that the operator shall hold an RPA Pilot License.”

7.1.2 When a RPA that is registered in Malawi is sold, the holder of the certificate of registration shall____

    - For aircraft that will be exported____

- Only the registered owner shall make submissions to the Authority (proof of identification of owner shall be evaluated). For corporate organizations legal documents (as prescribed by the Authority) attesting to said sale and authorization for deletion shall be included.

- The RPA to be exported shall be subject to physical inspection. Further guidance shall be prescribed by the Authority.

- An appropriate fee shall be charged by the Authority.

- The Certificate of Registration issued to the RPA shall be returned to the Authority.

- A deletion Certificate, where applicable, shall be issued to the importing State CAA.

- Old Registration marks shall be removed from the RPA before delivery to the prospective new foreign owner.

    - For RPAs that are not to be exported but changing ownership____

- For Companies, Organizations, Institutions, Associations and other entities, the Authority shall prescribe legal documentation that shall be submitted authenticating and supporting the application made.

- For individuals only the registered owner or legal proxy shall make application for change of Ownership. Documentation to be submitted shall be prescribed by the Authority.

- An appropriate fee shall be charged by the Authority.

    - All other conditions pertaining to sale of a RPA shall be prescribed by the Authority.

  - Identification Plate

    - Original Identification Plate

The original identification plate/data plate affixed by the manufacturer of the RPA shall not be tempered with. Where the Authority inspects a RPA and the inspection result show that the data plate has been tempered with, the owner or operator of such a RPA shall be guilty of an offence. The identification plate shall____

  - Contain information relating to the RPA type, model, serial number, marks of nationality and registration marks; and

  - Made of fireproof metal or other fireproof material of suitable physical properties; and

  - Secured to the RPA in a prominent position as the Authority shall prescribe:

    - An Identification Plate that has been previously removed

Where for any reason, maintenance or repair or accidental or rebuilding or retrofitting has to be effected, resulting in the original identification plate being interfered with____

- Approval from the Authority must be obtained prior to any action.

- On refitting the data plate, the Authority shall prescribe the manner in which the refitting will be done and issue documentation to capture the refitting

The following processes will be considered when evaluating documentation of a RPA for registration and certification purposes.

- DESIGN SPECIFICATION FOR RPA SYSTEMS

  - GENERAL ISSUES

The RPA comprises of airborne, ground based equipment and accessories mounted on the RPA. A high level of reliability of the entire RPA chain and low failure rate of critical parts of the RPA such as to prevent continued safe flight and/or recovery of the RPA including programmable emergency landing capacity (flight termination) will be a central feature of design of the RPA and subject to review during certification of RPA.

  - CRITICAL SYSTEMS

The critical systems that will be subject of review of a design of a RPA shall include but not limited to____

    - Flight controls;

    - Electrical systems;

    - Communication/data link;

    - Navigation systems;

    - Propulsion;

    - RPA control station; or

    - Flight termination.

  - DESIGN CRITERIA

    - FLIGHT CONTROLS

Flight control design should provide continuous control of the RPA by means of a controller unit whose display provides unambiguous operations and clear indications of RPA flight status.

    - ERGONOMICS AND HUMAN FACTORS

Ergonomics must encompass human factor principle in order to minimize human error.

    - FLIGHT INDICATIONS AND COLLISION AVOIDANCE

All flight indications and warnings, including collision avoidance, necessary to ensure safe control of the RPA flight path should be available in real time.

    - DEGRADATION AND FAILURE OF RPA

Degradation indications, including specific subsystem failure indication, should be available in the whole RPA particularly to the controller and in particular, the supervising controller who should be informed of any degraded mode of operations due to any failure.

    - SWITCHING OVER CAPABILITY

Capability to automatically switch to an alternate or degraded mode of operation must be available as a design feature.

    - CONTROL STATION DIAGNOSTICS CAPABILITY

The control station must have diagnostic and monitoring capability for the status of the RPA and real time, direct communications/surveillance, and continuous data transmission with high level of protection against hacking.

    - FLIGHT TERMINATION

Incorporation of a fail-safe Flight Termination System (FTS) or Autonomous Recovery System (ARS), which provides recovery to a predetermined recovery area with programmable capability for maintaining safe flight control or operation within design parameters. FTS or ARS shall be a mandatory feature for operations over or close to populous areas or where they will be within or close to controlled airspace.

No emphasis on a FTS or ARS will be required for those RPA operating in remote areas. Remote areas are those areas not within controlled airspace, 5nm of an aerodrome or an area with a dense population.

  - CERTIFICATION TECHNICAL ISSUES

    - FAIL SAFE PRINCIPLES, INDEPENDENCE AND REDUNDANCY

In order for a specific RPA design to be approved, especially for commercial activities, the design must exhibit proven fail-safe principles and application, system independence and adequate redundancy and back-up features to guarantee safe functioning of the RPA in the event of a system failure inclusive of critical systems. Redundancy of system management functions must be built into the system.

    - FAILURE DETECTION

RPA system design should provide for a failure detection unit/console (pre-flight and in-flight built-in-test) that will immediately notify the supervising controller of a system failure. The said design must cater for and minimise human error. An engineering analysis must form part of the design documents.

    - SOFTWARE

The Authority shall verify and validate RPA system software including for safety critical aspects.

    - FLIGHT MANAGEMENT SYSTEM

The RPA flight management system shall include the controls on the RPA controller, sensors, computers and actuation parts necessary to control the RPA. Any single failure of the flight control system should not affect the functions to control RPA recovery. Reversion to degraded modes of operation also should be incorporated into flight management system design. Continued control of the RPA should be possible in the event of a propulsion or power generation system failure.

    - ELECTRIAL SYSTEM

The specifications of any design of a RPA shall specify the maximum electrical load factor. The RPA electrical system should provide sufficient power and endurance to ensure safe operations and recovery throughout all phases of flight including in the event of an emergency. The system design must have capacity upon power failure, to set itself to a circular descent with a very shallow rate. For instance, if the battery system fails, the servos on alternating wings will be programmed to switch to failsafe, and will put the RPA into a Circular Pattern (30-40m) diameter and will descend at a very shallow approach until reaching the ground.

    - COMMUNICATIONS SYSTEM/DATA LINK

The Authority shall assign all frequencies used in RPA operations incorporation with Malawi Communication Regulatory Authority (MACRA) and other stakeholders. A RPA must have capability to continuously monitor Data link signal strength with maximum data link range cues being provided to the supervising controller.

The design must be such that any single failure of the communications system (uplink or downlink) must not affect normal control of the RPA.

    - NAVIGATION SYSTEM

The design of the RPA navigation system must meet the required navigation performance standards of the flight rules and the specific requirements for the airspace in which the operations are to be conducted. During flight planning, the Authority will evaluate more closely for flights to be conducted under IFR/IMC and in controlled airspace.

    - PROPULSION SYSTEM

The design of the RPA should ensure that all essential elements of the propulsion system meet required reliability standards and failure rates so as to achieve the Required Level of Safety.

    - RPA CONTROL STATION

The RPA control station may consist of a hand transmitter incorporating basic flight controls. Control stations for RPA operations Beyond Line of Sight should include controls and displays for aircraft attitude and performance, propulsion, navigation, aircraft systems and sensor operation as well as flight system and the flight data of the RPA shall be recorded at the ground station.

As part of the registration and certification process, the Authority will inspect and evaluate control station features against the requirements to assure the safety of air navigation of the RPA.

    - RPA STRUCTURE

A RPA aircraft structure shall be designed to withstand the maximum expected operational loads as determined by the intended operational flight envelope of the RPA. The Authority shall review structural designs to ensure safety and environmental aspects are adequately addressed.

    - FLIGHT TERMINATION SYSTEM

The design of a Flight Termination System (FTS) or Autonomous Recovery System (ARS) of a RPA shall incorporate an FTS/ARS which provides recovery to a predetermined recovery area. The FTS/ARS shall operate on demand or automatically following a failure to maintain safe flight control or operation within specified parameters.

  - RPA EQUIPMENT

During certification for safe flight under Instrument Flight Rules (IFR)/Instrument Metrological (IMC) conditions, the Authority will consider the following conditions but not limited to____

    - POSITION LIGHTS

The RPA shall have position lights installed in a manner as shall be approved by the Authority. The position lights shall be turned on at all times including during taxi, take off, flight and landing.

    - ANTI-COLLISION LIGHTS

The RPA shall have strobe lights installed in a manner prescribed by the Authority. The strobe lights shall be activated for launch, recovery and low level B-VLoS operations.

    - TRANSPONDER

For all RPA operations in controlled airspace, the RPA should have a serviceable Secondary Surveillance Radar (SSR) transponder installed. The supervising controller shall have the capability to turn the transponder on and off, manually select codes, and squawk and identification as directed, while the RPA is airborne.

    - RADIOS

The Operator shall have a two way radio and frequency allocated by the Authority. The supervising controller shall have full and immediate access to two way radios within the RPA control station as required to maintain communications. RPA communication architecture will be designed to allow the supervising controller to communicate with the ATC facilities and RPA ground crews controlling the RPA regardless of their location.

    - NAVIGATION SYSTEMS

The RPA shall have navigational equipment capable of displaying and reporting aviation information in a manner as is required by the Authority for RPA in controlled airspace. Navigational information should be available to the supervising controller in real time during the RPA operation.

    - RPA SYSTEM AND ATTITUDE DISPLAYS

The RPA must display to the supervising controller all RPA system and attitude information required for safe operation, control, and navigation.

    - FLIGHT RECORDER

The Authority may require that the RPA system have a recorder to record RPA systems and navigational status. This recorder will normally be installed within the RPA control station. No need for voice recording as it is only communicated from the ground and not the RPA. All RPA have full flight data recordings available on board.

    - BUILT-IN TEST EQUIPMENT (BITE) FUNCTION

The RPA shall have built in test function designed to exercise critical components, systems and provide an indication of their state of health inclusive of a pictorial or other display. The BITE information must be available to the ground station in real time during flight. The design and on board equipment shall have a set of diagnostic procedures to aid fault location. For in-flight use, the BITE will include remaining emergency power reserve.

IS16:11: ISSUANCE OF RPA LETTER OF APPROVAL (RLA)

- APPLICABILITY

This subpart prescribes requirements for issuance of RLA inclusive of accessory RPA equipment for RPA equipment registered in Malawi. Therefore, in terms of this subpart____

  - ISSUANCE OF RLA

The Authority shall issue an RLA, or other document, for RPA registered in Malawi after satisfactory evaluation of compliance with design, manufacture and specification data of the said RPA design certificate.

  - DURATION OF RLA

The duration of the RLA shall be effective as follows unless surrendered, suspended or revoked, or a special termination date is otherwise established by the Authority____

    - An RLA or Permit to Fly or some other similar document issued by the Authority, shall remain in effect or renewed, subject to the conditions and effective dates as shall be determined by the Authority for each category;

    - For Foreign RPA brought into Malawi for a limited period, a validation of an equivalent certificate to the RLA shall not extend beyond the validity of the foreign equivalent document but not exceeding 12 months.

    - Any occurrence (incident or accident), maintenance malpractices or other condition (verified by the Authority) deemed to render the RPA un-airworthy shall invalidate the RLA until the RPA is restored to an airworthy condition.

- ELIGIBILITY

  - APPLICATION FOR AN RLA

Any RPA registered owner in Malawi, or agent of the owner, shall apply for an RLA or other document as shall be prescribed by the Authority. The application for an RLA shall be in a form and manner acceptable to the Authority.

  - RPA IDENTIFICATION

Each application for an RLA shall show that the aircraft has the proper identification plates. The Authority shall prescribe some indication as to how the RPA needs to be registered and which designation it should use.

  - CLASSIFICATION OF RLA

The Authority shall classify the RLA or other document in the following categories____

- Sport and Recreation valid for a period up to 3 years;

- Commercial Operations in Health, Agriculture, Mining, Wildlife Management, Mapping, Filming and others as shall be determined by the Authority for a period up to 1 year unless otherwise exempted by the Authority; or

- Special Category. (Note: This category shall be for research and development or any other activity as deemed necessary by the Authority)

- RPA “SYSTEM SAFETY” TO CONSIDER WHEN APPLYING FOR RLA

An application for an initial approval, where no certification exists from an ICAO Contracting State, shall be accompanied by all the following information that is available from the manufacturer____

(a) The RPA Operating Manual from the manufacturer; and

(b) A submission, prepared by the applicant outlining all of the following information to the extent known, or available____

      - For Class 1 and 2 operations, only section 1 and 2 below; or

      - For Classes 3, 4, 5, 6, 7 and 8 all sections below:

SECTION 1: RPA INFORMATION

  - RPA type

  - RPA Structure

  - RPA composition

  - Flight envelope capability

  - RPA dimensions/measurements and mass together with drawings

  - Mass and balance

  - Payloads (specific or generic)

  - Use of frequencies

  - Remote pilot station

  - Ground support equipment

  - Flight support system

- SECTION 2: PERFORMANCE CHARACTERISTICS

  - Maximum altitude

  - Maximum endurance

  - Maximum range

  - Airspeed (take-off, cruise, landing, stall maximum)

  - Maximum rate of climb

  - Maximum rate of descent

  - Maximum bank angle

  - Turn rate limits

  - Propulsion system (i.e. engine/motor, fuel, electrical, hydraulic, pneumatic, gas, solar)

- SECTION 3: PERFORMANCE CAPABILITIES AND LIMITATIONS

  - RPA performance limitations due to environmental and meteorological conditions (wind, ice, humidity, temperature, precipitation, hail).

  - Required take-off and landing distances and/or areas

  - Flight control surfaces and actuators

  - Location of all air data sensors, antennas, radios and navigation equipment with respect to segregation and redundancy.

  - Autopilot type, manufacturer (description or working method)

  - Navigation systems (description of the components, together with horizontal, vertical position and velocity accuracy)

  - Sensors and/or telemetry

- SECTION 4: EMERGENCIES AND SYSTEM FAILURES

At the minimum, the following emergency scenarios should be documented, with procedures for handling them____

  - Loss of autopilot

  - Loss of flight control due to servo failure, if applicable

  - Loss of propulsion power

  - Loss of engine power (one engine out), if applicable

  - Low battery voltage, if applicable

  - Loss of navigation components (heading or altitude)

  - Loss of Global Navigation Satellite System

  - Loss of data link (radio control link failure)

  - Loss of remote pilot station (remote pilot station communication failure)

  - Loss of power of remote pilot station

  - Loss of remote pilot/RPA observer communication

  - Dealing with structural damage

4.13 Any other failure modes or scenarios other than those listed above that can endanger safe flight, shall be identified, described and managed in a safe manner.

- SECTION 5: HAZARD ASSESSMENT

An objective assessment of the RPA’s potential hazard considerations, which should include-

  - Identification of RPA functions

  - Systems that assist with the identification of failure conditions

  - Management and mitigations of the failure conditions

  - A list of alarms and methods for troubleshooting

- SECTION 6: FAIL-SAFE FEATURES

Procedures to be followed by the remote pilot in case of malfunctions or failure. Information of flight termination features.

IS16:16 REPORTING OF INCIDENTS AND ACCIDENTS RELATING TO RPA OPERATION

| OCCURRENCE REPORT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Attachment ‘A’ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (DCA Headquarters) |  |  |  |  |  |  |  |  |  |  |
| Department of Civil Aviation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | To be sent to: Flight Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Email |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PLEASE COMPLETE IN BLACK INK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Complete all sections where information is relevant. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date received by DCA |  |  |  |  |  |  |  |  |  |  |  | DCA Occurrence No. |  |  |  |  |  |  |  |  |
|  |  | For multi-choice boxes, indicate which entry is appropriate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Aircraft Type and Series |  |  |  |  |  |  | Registration |  |  |  |  |  | Operator |  |  |  |  |  |  | Date of Occurrence |  |  |  |  |  |  | Flight Phase |  |  |  |  | Nature of Flight |  |  |  |  |
|  | 1 |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  | 4 |  |  |  |  | 22 |  |  |  |  |  |  | 23 |  |  |  |  |
|  |  |  |  |  |  |  |  | FLIGHT AND WEATHER DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PARKED |  |  |  |  |  |  | PAX |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TAXYING |  |  |  |  |  |  | FREIGHT |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TAKE-OFF |  |  |  |  |  |  | SURVEY |  |  |  |  |
|  |  | Flight No. |  | DAY |  |  |  |  | Wind | Runway |  |  |  | Precipitation |  |  |  |  |  |  |  | Icing |  | Turbulence |  |  | INIT CLIMB |  |  |  |  |  |  | PLEASURE |  |  |  |  |
|  | 5 |  |  |  |  |  |  | 12 |  | Used |  |  | 18 |  |  |  |  |  |  | 19 |  |  |  | 20 |  |  | CLIMB |  |  |  |  |  |  | AGRICULTURAL |  |  |  |  |
|  |  |  |  | NIGHT |  |  |  |  |  | 16 |  |  | RAIN |  |  |  | LIGHT |  |  |  |  |  |  | LIGHT |  |  | CRUISE |  |  |  |  |  |  | BUSINESS |  |  |  |  |
|  |  | From |  |  |  |  |  |  | IAS |  |  |  | SNOW |  |  |  | MOD |  |  | LIGHT |  |  |  | MOD |  |  | DESCENT |  |  |  |  |  |  | CLUB/GROUP |  |  |  |  |
|  |  |  |  | TWILIGHT |  |  |  |  |  | State |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | kts |  |  |  | FLEET |  |  |  | HEAVY |  |  | MOD |  |  |  | SEVERE |  |  | HOLDING |  |  |  |  |  |  | PRIVATE |  |  |  |  |
|  | 6 |  |  | 9 |  |  |  | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 17 |  |  | HAIL |  |  |  |  |  |  | HEAVY |  |  |  | EXTREME |  |  | APPROACH |  |  |  |  |  |  | POSITIONING |  |  |  |  |
|  |  | To |  |  | Time |  |  | HT/ALT/FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LANDING |  |  |  |  |  |  | FERRY |  |  |  |  |
|  | 7 |  |  | 10 |  |  |  | 14 | ft | DRY |  |  |  |  |  |  |  | Cloud Type |  |  |  |  |  |  |  |  | CIRCUIT |  |  |  |  |  |  | TEST |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | WET |  |  |  |  |  |  |  |  | Height/ft |  |  |  |  |  |  |  | AEROBATICS |  |  |  |  |  |  | TRAINING |  |  |  |  |
|  |  | Geog. Position |  |  | Visibility |  |  |  | OAT | ICE |  |  |  |  |  |  |  | Amount/8ths |  |  |  |  |  |  |  |  | HOVER |  |  |  |  |  |  | PARACHUTING |  |  |  |  |
|  | 8 |  |  | 11 |  |  |  | 15 | °c | SNOW |  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOWING |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | SLUSH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NARRATIVE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Continue on back |  |  |  |  |
|  |  | ENGINEERING DETAILS |  |  | Aircraft Constructor’s No. |  |  |  |  |  |  |  | Engine Type & Series |  |  |  |  |  |  |  |  |  |  | Ground Phase |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maintenance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ground Handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Taxy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Unattended |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 25 |  |  |  |  |  |  | 26 |  |  |  |  |  |  |  | 27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Component/Part |  |  |  | Location on aircraft |  |  |  |  |  |  | Manual Reference |  |  |  |  |  |  |  |  | Maintenance Organisation |  |  |  |  |  |  |  | 33 |  | Maintce Prog |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | O.C. |  |  |  | C.M. |  |  | H.I. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tel. No. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  |  |  | 29 |  |  |  |  |  |  | 30 |  |  |  |  |  |  |  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Manufacturer |  |  |  |  |  | Part No. |  |  |  |  |  | Serial No. |  |  |  |  |  |  |  |  |  |  | TOTAL |  |  |  | Since O/H |  |  |  |  | Since |  |  | Manufacturer |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HOURS/ |  |  |  |  |  |  |  |  |  | or repair |  | Inspection |  |  |  |  | Advised |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CYCLES/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LANDINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34 |  |  |  | 35 |  |  |  |  |  |  | 36 |  |  |  |  |  |  |  | 37 |  |  |  |  | 38 |  |  |  | 39 |  |  | 40 |  |  |  | NO |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 41 |  |  |
|  |  | Is there any published |  |  |  |  |  |  |  |  |  |  | Reference No. and Compliance Status of Aircraft or Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Airworthiness Information or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | control procedures (e.g. AD, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SB etc) relevant to occurrence |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 42 | YES | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Report |  |  | If report is submitted Voluntarily i.e. not subject to |  |  |  |  |  |  |  |  |  |  |  |  | Organisation |  |  |  |  |  |  |  |  |  |  |  | Address and Tel. No. |  |  |  |  |  |  |  |  |
|  |  | ORIGINAL | SUPPL. |  |  |  |  | mandatory requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (if reporter wishes to be contacted privately) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  |  |  |  |  |  |  |  |  |  |  |  |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Reporter’s Investigation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NIL OPEN | CLOSED | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  |  | Can the information be disseminated in the interest of |  |  |  |  |  |  |  |  |  |  | 49 |  |  |  |  |  |  |  |  |  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flight Date |  |  | safety |  |  |  |  |  |  |  |  |  |  |  | Reference No. |  |  |  |  |  | Date |  |  |  |  | Name |  |  |  |  |  |  | Signature |  |  |  |  |
|  | Record Held |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 45 | YES | NO | 47 |  |  |  | YES |  | NO |  |  |  |  | 50 |  |  |  |  |  | 51 |  |  |  | 53 |  |  |  |  |  |  |  |  |  |  |  |  |  |

IS16:17: AUTHORISATION FOR RPA OPERATIONS

Unless otherwise specified by the Authority the request for authorisation shall include the following____

- Name and contact information of the operator;

- RPA characteristics (type of aircraft, maximum certificated take-off weight, number of engines, wing span);

- Copy of Certificate of Registration;

- Aircraft identification to be used in radiotelephony, if applicable;

- Copy of the Certificate of Airworthiness, if applicable;

- Copy of the Remote Pilot(s) Licence;

- Copy of the Aircraft Radio Station Licence, if applicable;

- Description of the intended operation (to include type of operation or purpose), flight rules, Visual Line of Sight (VLoS) operation if applicable, date of intended flight(s), point of departure, destination, cruising speed(s), cruising level(s), route to be followed, duration/frequency of flight;

- Take-off and landing requirements;

- RPA performance characteristics, including:

  - Operating speeds;

  - Typical and maximum climb rates;

  - Typical and maximum descent rates;

  - Typical and maximum turn rates;

  - Other relevant performance data (e.g. limitations regarding wind, icing, precipitation); and

  - Maximum aircraft endurance;

- Communications, navigation and surveillance capabilities____

  - Aeronautical safety communications frequencies and equipment, including____

    - ATC communications, including any alternate means of communications;

    - Command and Control links (C2) including performance parameters and designated operational coverage area; or

    - Communications between remote pilot and RPA observer, if applicable.

  - Navigation equipment; and

  - Surveillance equipment (e.g. SSR transponder, ADS-B);

  - Detect and avoid capabilities;

  - Emergency procedures, including____

    - Communications failure with ATC;

    - C2 failure; or

    - Remote pilot/RPA observer communications failure, if applicable;

  - Number and location of remote pilot stations as well as handover procedures between remote pilot stations, if applicable;

  - Document attesting noise certification, if applicable;

  - Confirmation of compliance with the Aviation (Airport Security) Regulations;

  - Payload information/description; and

  - Proof of adequate insurance coverage.

- Where documents identified in IS 17 above are issued in a language other than English, the RPA operator shall ensure that an English translation is included.

IS16:20: REQUIREMENTS FOR THE ISSUE OF AN RPL

- Knowledge and Skill Requirements

    - An applicant for a remote pilot certificate shall have demonstrated a level of knowledge appropriate to the privileges granted to the holder of the certificate, in the following;

- Air Law for Remotely Piloted Aircraft (RPA)

    - Aviation (Remotely Piloted Aircraft) Regulations and the associated Implementing Standards

- RPA General Knowledge

- Human Factors

    - Vision____

      - Empty field myopia;

      - Adaptation to darkness;

      - Auto kinesis;

    - Stress management (Causes of stress)

    - Signs of any symptoms of fatigue.

- Meteorology____

    - Factors affecting air density

    - Fog and mist

    - Wind and gusts

    - Thunderstorms

    - Aviation weather reports.

- Navigation____

    - Latitude and longitude

    - Aeronautical chart information (such as VFR)

    - GPS (components of a GPS system as used on an RPA)

- Operational Procedures.

- Principles of Flight related to RPA.

- Radiotelephony

- The applicant for remote pilot licence must pass the knowledge examination before taking the skill test.

- The applicant for a skill test shall obtain a recommendation from an authorized instructor or approved training organisation responsible for the training upon completing the training.

- An applicant for a remote pilot certificate shall pass a skill test to demonstrate the ability to perform, as remote PIC of the appropriate RPA category and associated RPS, the relevant procedures and manoeuvres with the competency appropriate to the privileges granted. This ability includes;

- Recognise and manage threats and errors;

- Operate the RPA within its limitation or those limitations imposed by regulations;

- Complete all manoeuvres with smoothness and accuracy;

- Exercise good judgements and airmanship;

- Apply aeronautical knowledge; and

- Maintain Control of the RPA at all times in a manner such that the successful outcome of a procedure or manoeuvre is assured.

- Credit

A holder of a licence issued by the Authority may be credited towards the requirements for theoretical knowledge instruction and examination requirements for the remote pilot certificate.

- Passing Grade

The Authority shall prescribe the minimum passing grade.

- Retesting after failure____

    - A candidate may not apply to rewrite an examination until he or she has received the official result notification.

    - A candidate who has failed an examination conducted by the Authority for the issues of RPL may not rewrite the examination:

Where a mark of less than 50% was achieved, within a period of 2 calendar months.

    - If a mark of less than 50% is achieved in conjunction with a third or subsequent failure, the respective periods of 2 calendar months shall run concurrently.

- Re-mark after failure____

    - A candidate who fails an examination with a mark of 65% or above may, within 30 days from the date of notification of the examination results, apply in writing for a re-mark.

    - The application shall be accompanied by the appropriate fee as prescribed.

    - If the re-mark results in a pass of the examination concerned, the re-marking fee will be refunded.

    - A candidate may not rewrite any examination in respect of which a re-mark has been requested and for which the candidate has not been notified of the official result. Failure to comply with this condition will result in the premature re-write being invalidated and the applicable examination fee forfeited.

- RPA Instructor

- Prior to the issuance of an RPA instructor authorisation by the Licensing authority, an RPA instructor applicant should____

- Hold a remote pilot licence with appropriate category, class and type rating(s) for which the privilege to instruct is being sought;

- Have sufficient training and experience to attain the required level of proficiency in all of the required tasks, manoeuvred, operations and principles and methods of instructions; and

- Be entitled to act as PIC of the RPA during the RPA instruction.

- Training programmes for the RPA instructor role should focus on development of the competencies in the following area____

- Manage safety;

- Prepare the training environment;

- Manage the trainee;

- Conduct training;

- Perform trainee assessment;

- Perform course evaluation; and

- Continuously improve performance

- An applicant for an RPA instructor authorisation should pass an assessment of competence in the relevant type or class of RPA to demonstrate the ability to instruct a student remote pilot to the level required for the issue of the remote pilot licence.

- All RPA instructors shall receive refresher training and re-assessed within a period of not more than 3 years in accordance to the documented training and assessment training acceptable to the licensing authority and implemented by the a certificated or approved organization.

- Special Conditions

In the case of introduction of new RPA or RPS in an operator’s fleet, when compliance with the requirements established by the Authority is not possible, the Authority may consider issuing a specific authorization giving privileges for RPAS instruction. Such an authorization should be limited to the instruction flights necessary for the introduction of the new type of RPA or RPS.

The validity period for this authorization shall be for the instruction sought only.

- Revalidation and renewal of RPA instructor rating

  - When issuing further RPA instructor authorisation, and for the purpose of revalidation, the Licensing Authority should credit____

- Applicants with teaching and learning skills already demonstrated for the RPA instructor authorisation held; or

- Hours in the role of an RPA examiner during skill test or proficiency checks in full towards revalidation requirements for all RPA instructor authorisations held.

  - The validity period of RPA instructor authorisation is 3 years and renewal is subject to attendance of refresher training and re-assessment check within the period assigned.

- Night Operation

Licensing Authority shall require remote pilots to receive dual instruction in RPA night  
 operations including take-off, landing and navigation before exercising the privilege of the 
 remote pilot licence at night

- Medical Assessment

- The Class three medical assessments for B-VLoS operations or operations involving RPA classified as Class 3 or higher.

For all other classes or types of operation, involving RPA classified as Class 2 or lower, submit a self declared medical assessment report by the aviation medical examiner.

- The period of validity of the medical assessment shall be 24 months from the date of medical examination and reduced to 12 months when licensed holder has passed their 40th birthday.

- Flight Training

9.1   GENERAL

Flight training can be a combination of simulator and aircraft training. The exercises need not be done in the sequence given.

  - ITEMS APPLICABLE TO ALL REMOTE PILOT LICENCES

  - Aircraft pre-flight inspection and setup;

  - Post-launch in-flight evaluation procedures (checking of systems directly after launch-if applicable to the category of licence);

  - Automated flying and flight controller flight modes;

  - First person view (FPV) Flying (if applicable);

  - Parachute-assisted landing (if applicable to the category of licence);

  - Evasive action (maneuvers) to avoid collisions; and

  - Post-flight inspection.

  - ITEMS APPLICABLE TO THE REMOTE PILOT LICENCE (AEROPLANE) (RPL(A))

  - Climbing and descending;

  - Turns while maintaining altitude;

  - Climbing and descending turns;

  - Speed changes while maintaining altitude;

  - Horizontal figure eight;

  - Stalls;

  - Takeoffs;

  - Catapult launch (if applicable);

  - Hand launch (if applicable);

  - Approaches and landings;

  - Engine failure____

    - At altitude;

    - After takeoff; or

    - On the approach.

  - VTOL – Vertical takeoff and landing (if applicable)

  - ITEMS APPLICABLE TO THE REMOTE PILOT LICENCE (HELICOPTER) (RPL(H))

  - Tail-in hover;

  - Tail-in hover performance squares and circles;

  - Takeoffs;

  - Tail-in hover performing a horizontal figure eight;

  - Tail-in hover performing a vertical rectangle;

  - Side-on hover (both sides);

  - Transition from hover to forward flight;

  - Transition from forward flight hover;

  - Climbing and descending from level flight;

  - Approach and landing;

  - Nose-in hover; and

  - Autorotation.

  - ITEMS APPLICABLE TO THE REMOTE PILOT LICENCE MULTI-ROTOR (RPL(MR))

  - Tail-in hover;

  - Tail-in hover yawing slowly to right and left;

  - Tail-in hover, move to right then backwards;

  - Tail-in hover, move forwards then backwards;

  - Tail-in hover, ascend and descent;

  - Takeoffs;

  - Tail-in hover performing a horizontal rectangle;

  - Tail-in hover performing a vertical rectangle;

  - Nose-in hover;

  - From hover fly a square box rotating (yawing) the multi-rotor in the directions of flight;

  - From hover fly a circle rotating (yawing) the multi-rotor nose-in to the centre of the circle;

  - Transition from hover to forward flight;

  - Climbing and descending from level flight;

  - Turns from level flight;

  - Speed control in the level flight;

  - Approach and landings; and

  - Actions after failure of a motor.

IS 16:47 FLIGHT FOLIO

The Flight Folio shall incorporate the following information for each flight____

- Aircraft registration

- Date

- Name(s) of RPA crew members

- Duty assignment of RPA crew members

- Place of departure

- Place of arrival

- Time of departure (off-block time)

- Time of arrival (on-block time)

- Hours of flight

- Nature of flight

- Incidents/observations (if any)

- Signature of RPA pilot

- The current maintenance statement giving the aircraft maintenance status of what maintenance, scheduled or out of phase, is due;

- All outstanding deferred defects which affect the operation of the aircraft;

- Fuel and oil used (if applicable)

- Fuel and oil uplift (if applicable)

- Battery charge status at the beginning and end of the flight (if applicable)

OPERATINAL GUIDELINES FOR RPA CLUBS

IS16:53: RECREATIONAL AND SPORTS RPA OPERATIONS

The following requirements shall apply to RPA clubs intending to operate for sport and recreation.

- GENERAL PROVISIONS

  - An RPA club shall be registered in accordance with the provisions pertaining to the registration of clubs in Malawi for it to be recognised and approved by the Authority.

  - No RPA club shall operate without the approval by the Authority.

  - The RPA club is required to develop an Operational Manual that provides for____

    - Membership requirements;

    - Administration of members;

    - Training requirements for its members;

    - Procedures and guidelines of operations;

    - Types of operation;

    - Class of equipment operated;

    - Security arrangement for operations; and

    - Reporting mechanisms of incidents and accidents of the RPA operations.

ADMINISTRATION OF THE CLUB

  - The club management shall ensure that members____

    - Have adequate training to facilitate operations;

    - Are informed on current regulations, policies and procedures;

    - Adhere to safe business practices in their activities;

    - Are knowledgeable of airspace restrictions that apply in the area of operation as approved; and

    - Are conversant with and meet the training requirements of the club.

RESPONSIBILITY OF THE CLUB MANAGEMENT

  - The administrator of the club shall____

    - Obtain consent of the property owner or person in charge of the area of operation.

    - Ensure that the club’s recognition status is current with the Authority

    - Develop and operationalise a training programme and plan for their membership;

    - A current list of members and particulars of their RPA.

    - Maintain a record/database of all accidents and incidents that occur within their area of jurisdiction;

    - Ensure that it has adequate personnel and are properly qualified and competent to perform their allocated tasks and responsibilities;

    - Have procedures for responding to an incident, accident, medical emergency, or if any RPA becomes uncontrollable.

    - Immediately stop all operations if unable to meet the exemption requirements or if the safety of a person, property or other aircraft is at risk;

    - Ensure that club activities do not interfere with civil aviation;

    - Adhere to laws from all levels of government; and

    - Inspect their RPA on site before conduct of any flight to ensure that they are safe.

IS16:63: OPERATIONS MANUAL

Each ROC holder shall submit to the Authority for approval, an Operational Manual, Flight Manuals and maintenance manuals whose content commensurate with the size and scope of their intended operation. An Operations Manual shall include each item set forth below, unless otherwise approved by the Authority.

PART A: GENERAL

- INTRODUCTION

  - Purpose and scope of manuals

  - A statement that the manual complies with all applicable Authority regulations and requirements and with the terms and conditions of the applicable RPA operator certificate.

  - A statement that the manual contains operational instructions that are to be complied with by the relevant personnel in the performance of their duties.

  - List of manuals comprising operations manual

  - A list and brief description of the various operations manual parts, their contents, applicability and use.

  - Responsibility for manual content.

  - Responsibility for manual amendment.

  - List of effective pages.

  - Distribution of manuals and amendments.

- SAFETY MANAGEMENT SYSTEM

  - Safety Policy;

  - Description of safety management system; and

  - Accident and Investigation policies.

- QUALITY SYSTEM

  - Description of quality system adopted

- MANAGEMENT ORGANISATION

  - A description of the organisational structure including the general company organisation and operations department organisation. The relationship between the operations department and the other departments of the company. In particular, the subordination and reporting lines of all divisions, departments, etc., which pertain to the safety of the RPA operations, shall be shown.

  - Accountable Manager -duties and responsibilities.

  - Nominated personnel - functions duties and responsibilities.

  - RPA Pilot - duties and responsibilities.

  - Support personnel in the operation of RPA – duties and responsibilities

  - A description of the objectives, procedures and responsibilities necessary to exercise operational control with respect to flight safety.

- DOCUMENTATION

  - Documents required in RPA operations

  - Document storage and retention period

PART B: RPA OPERATING INFORMATION

- CREW INFORMATION

  - Flight team/crew composition

  - Qualification requirements of RPA pilot and support crew

  - Medical competencies

  - Operations of different types of RPA

- OPERATIONS PROCEDURES OF RPA

  - Operating Limitations and conditions____

    - Scope and feasibility

    - Site location assessment considerations____

      - Airspace considerations

      - Conflicting aircraft or RPA traffic

      - Hazards identification

      - Local by-laws

      - Obstructions

      - Restrictions

      - Habitation and conflicting activities

      - Public access

      - Permission from landowner

      - Likely operating site and alternative sites

      - Weather conditions and planning

  - Notification of intended operations to affected persons

  - Communications procedures

  - On site procedures and pre-flight checks____

    - Site visual survey

    - Selection of operating area

    - Crew briefing

    - Cordoning off procedure (if applicable)

    - Communication range and capability requirements

    - Weather observations

    - Re-fuelling or recharging

    - Loading of operational equipment

    - Preparation and assembly of RPA on site

    - Pre-flight and post flight checks

- RPA FLIGHT MANAGEMENT

  - Assembly and functional checks

  - Pre-flight checks

  - Normal flight procedures associated with relevant systems____

    - Start

    - Take-off

    - In flight

    - Landing

    - shutdown

  - Inflight checks associated with relevant systems

  - Abnormal procedures associated with relevant systems

  - Emergency Procedures associated with relevant systems____

    - Unique to the RPA to be operated

    - Fire-Risk and preventative measures

    - Accidents considerations and emergency response plan

    - Loss of control link

- RPA USER MANUAL

- SECURITY

An operator shall ensure that policy and procedures in respect of the following aspects of security are addressed in its Operations Manual____

  - RPA operator organisation and designation of a security coordinator;

  - Requirements for checks and searches of specific areas and accessible compartments of the interior and exterior of RPA;

  - Prevention of unauthorised access to remotely piloted aircraft and ground control stations;

  - Protection efforts pertaining to limiting the software and C2 links from forms of interference;

  - Response procedures for crew members and other staff or threats and incidents;

  - Special procedures for____

    - crop spraying operations

    - Carrying of weapons

    - Dangerous goods

    - High consequence dangerous goods and high value cargo (if applicable)

  - Crew member briefings concerning security/safety sensitive cargo loads

  - Additional security measures for special or more threatening situations

  - Reporting of security relating incidents to the Authority

  - Details on procedures and frequency on conducting background checks and recurrent criminal record checks; and

  - Details on security awareness and response procedure training.

PART C: AREAS, ROUTES AND AERODROMES

- Areas of Operations

- Operating site planning and assessment

- Authorizations including site permissions

PART D: TRAINING

- Training syllabi and checking programs for RPA crew

- Training syllabi and checking programs for RPA support crew

- Training syllabi and programs for personnel other than crew

- Recurrent training programs

- Additional training requirements that individual clients specify for the proposed operations.

MAINTENANCE

IS16:67: CONTINUED SYSTEM MAINTENANCE

No person may operate an RPA above a weight category specified by the Authority, unless the RPA and its components____

- REQUIREMENT FOR A MAINTENANCE AND INSPECTION PROGRAM

All RPA systems are maintained in accordance with a maintenance and inspection program established by the owner or operator and approved by the Authority.

- MAINTENANCE PROGRAM

The maintenance program shall include a description of the RPA and components and recommended methods for the accomplishment of maintenance tasks. Such information shall include guidance on defect diagnosis.

- MAINTENANCE TASKS

The maintenance program shall include the maintenance tasks and the recommended intervals at which these tasks are to be performed. Maintenance tasks and frequencies prescribed by the manufacturer shall be considered mandatory by the Authority.

- MAINTENANCE RELEASE

An RPA maintenance program shall have a maintenance release process which shall comprise signed documentation performing the maintenance in a manner that will be satisfactory to the Authority. The maintenance release shall indicate the maintenance performed and level of completion and success. A maintenance release shall contain certification details inclusive of____

  - Basic details of the maintenance carried out;

  - Date such maintenance was completed;

  - When applicable, the approved maintenance organisation; and

  - The identity of the person or persons signing the release.

- INSPECTION PROGRAMS

An RPA of specific weight category and/or use shall have an inspection program. The owner or operator of a RPA shall compile the inspection program based on the design, specifications and manufacturer data. The program shall be approved by the Authority.

- WHERE MAINTENANCE SHALL BE CONDUCTED

An RPA maintenance including its components shall be conducted under a maintenance facility or other arrangement as shall be approved by the Authority.

- MAINTENANCE AND INSPECTION RECORDS AND ENTRIES

Each person who maintains, performs preventive maintenance, rebuilds, or alters an aircraft or life limited parts shall, when the work is performed satisfactorily, make an entry in the maintenance record of that equipment as follow:

  - DESCRIPTION OF WORK

A description (or reference to data acceptable to the Authority) of work performed, including____

    - The total time in services (hours, calendar time and cycles, as appropriate) of the aircraft and all life-limited components;

    - The current status of compliance with all mandatory continuing airworthiness information;

    - Appropriate details of alterations and repairs;

    - Time in service (hours, calendar time and cycles, as appropriate) since last overhaul of the RPA or its components subject to a mandatory overhaul life;

    - The current status of the aircraft’s compliance with the maintenance program; and

    - The detailed maintenance records to show that all requirements for signing of a maintenance release have been met.

  - DETAILS OF COMPLETION OF WORK

Completion of work details shall include the date when the work was performed.

  - DETAILS OF PERSONS CONDUCTING THE WORK

The maintenance entry shall indicate the following____

    - Name, signature, certificate number, and kind of license held by the person approving the work.

    - Major repairs and alterations shall be entered on a form by the person performing the work in a manner and format as shall be prescribed by the Authority.

- WEIGHING OF RPA AND LISTING OF RPA EQUIPMENT

The Authority shall prescribe the MTOW and/or use of RPA that shall be weighed. The Authority shall also specify the form and manner the weighing shall be conducted, the various issues to be recorded and list of equipment to be attached to the weight schedule.

An RPA payload should be able to be weighed individually.

RPA registration shall be in the following weight categories____

- Class 1 – less than 5kg;

- Class 2 – 5 to 25kg; and

- Class 3 –  Above 25kg.

IS 16:70 RMT LOGBOOK

The log book shall contain the following information as a minimum____

- Full name;

- Identification number;

- Name of employer;

- Record of all technical courses attended;

- Date of maintenance;

- Type and make of RPA (e.g. fixed wing or multi-rotor);

- Work carried out (inspection, repair, overhaul etc); and

- Signature of Quality Assurance Personnel.